Standards for Effluent Discharge Regulations

- General Notice No.44.of 2003

- THE ENVIRONMENT PROTECTION ACT 2002

Regulations made by the Minister under sections 39 and 96 of the Environment Protection Act 2002

1. These regulations may be cited as the Environment Protection (Standards for effluent discharge) Regulations
 2003.

2. In these regulations -

(b)“effluent” means water sullied or contaminated by any matter, in solution or suspension and derived from the use of the water in connection with domestic, industrial or other activities;

“HWM” means the High Water Mark at spring tide;

“influent” means water diverted from a river, stream, spring, canal, underground or water supply network used in connection with any activity listed in the First Column of the First Schedule;

“parameter” means, in relation to an effluent, the characteristics or constituent elements set out in the Second Column of the First Schedule in respect of the corresponding activity set out in the First Column of the First Schedule;

“Wastewater system” –

(a) means a sewer, conduit, pump, engine or other appliance used or intended to be used for the reception, conveyance, removal, treatment and disposal of effluent; and

(b) does not include house sewers;

“waterbody” includes a stream, a river, a canal, a lake, a pond, a reservoir, an estuary, a wetland and
 underground water;

“watercourse” means any natural or artificial channel, pipe or conduit, excluding the sewerage system,
 carrying, or that may carry, and discharging water directly or indirectly into a water body;

3.No person shall discharge effluent onto land, into a watercourse or into a waterbody unless he ensures that the
 parameters of the effluent do not exceed the permissible limits set out in the Second Schedule.

4.Notwithstanding regulation 3 or any other enactment, no person shall discharge or cause to be discharged any
 effluent into a waterbody or watercourse used or earmarked to be used for potable water supply.

5.Notwithstanding regulation 3, any person using an influent, the limits concentration or value of the any
 parameters of which exceeds the permissible limit for that parameter set out in the Second Schedule, shall
 ensure that the concentration or value of the parameters of in the effluent does not exceed those that of the
 influent.

6.Any industry existing prior to the promulgation of these regulations and which is within a distance of 200 metres from the HWM shall comply with the permissible limits set out in the Third Schedule.

7.These regulations shall come into operation on 01 September 2003.

Made by the Minister on 05 February 2003

FIRST SCHEDULE

(regulation 2)

- List of parameters for each industrial activity

| - INDUSTRIAL ACTIVITY | - PARAMETERS |
| --- | --- |
| - Textile manufacturing | Colour, Temperature, pH, COD, BOD5, Reactive Phosphorus, Free Chlorine, TSS, Chloride, Sulphate, Sulphide, Ammoniacal Nitrogen, Nitrate as N, Detergents, Cadmium, Total Chromium, Cobalt, Copper,Molybdenum, Sodium, Zinc, Oil & Grease, Total Pesticides, Total Organic Halides. |
| Metal Plating & Galvanising | Temperature, pH, COD, Free Chlorine, TSS, Chloride, Sulphate, Sulphide, Nitrate as N, Cyanide, Cadmium, Total Chromium, Cobalt, Copper, Iron, Lead, Nickel, Zinc, Oil & Grease, Total Organic Halides. |
| - Slaughtering | Temperature, pH, COD, BOD5, TSS, Chloride, Nitrate as N, TKN, Oil & Grease, Total Coliforms, E. Coli |
| Canning & Food Processing | Temperature, pH, COD, BOD5, Free Chlorine, TSS, Chloride, Nitrate as N, TKN, Sodium, Oil & Grease, Total Coliforms. |
| Dairy Processing | Temperature, pH, COD, BOD5, TSS, Selenium, Oil & Grease, Detergents, Ammoniacal Nitrogen. |
| Soft Drink Bottling | Temperature, pH, COD, BOD5, TSS, Sodium, Zinc, Detergents. |
| Breweries & Distilleries | Temperature, pH, COD, BOD5, TSS, Nitrate as N, Selenium, Zinc, Oil & Grease, Detergents, Ammoniacal Nitrogen. |
| Laundry processes | Temperature, pH, COD, BOD5, Reactive Phosphorus, Free Chlorine, TSS, Nitrate as N, Total Chromium, Copper, Iron, Lead, Oil & Grease, Total Organic Halides, Detergents |
| Edible Oil Refining | Temperature, pH, COD, BOD5, TSS, Chloride, Sodium, Oil & Grease, Total Organic Halides, Phenols, Detergents. |
| Paint Manufacturing | Colour, Temperature, pH, COD, BOD5, TSS, Chloride, Sulphate, Sulphide, Aluminium, Cadmium, Total Chromium, Cobalt, Copper, Lead, Mercury, Molybdenum, Zinc, Oil & Grease, Total Organic Halides. |
| Mechanical Workshop | pH, COD, BOD5, Oil & Grease, Total Chromium, Lead, Manganese, Zinc. |
| Thermal Power Plant | Temperature, pH, TSS, Oil & Grease, Total Chromium, Copper, Iron, Zinc. |
| Soap & Detergents Manufacturing | Temperature, pH, COD, BOD5, Reactive Phosphorus, Free Chlorine¸ TSS, Oil & Grease, Total Organic Halides, Detergents, Ammoniacal Nitrogen. |
| Manufacture of Pharmaceutical products | Temperature, pH, COD, BOD5, Reactive Phosphorus, TSS, Sulphide, Oil & Grease, Phenols and Detergents. |
| Tanning | Colour, Temperature, pH, COD, BOD5, Reactive Phosphorus, TSS, Sulphate, Sulphide, Nitrate as Nitrogen, Cadmium, Total Chromium, Mercury, Oil & Grease, Total Organic Halides, Total Coliforms, E. Coil Coli, Ammoniacal Nitrogen. |
| Manufacture of Chemical Fertilizers | Temperature, pH, COD, BOD5, Reactive Phosphorus, TSS, Sulphate, Oil & Grease, Ammoniacal Nitrogen. |
| Livestock Breeding | pH, COD, BOD5, Reactive Phosphorus, TSS, Nitrate as Nitrogen, TKN, Total Coliforms, E. Coli, Ammoniacal Nitrogen. |

- SECOND SCHEDULE

(regulation 4)

Effluent discharge Standards

| Parameter | Unit | Maximum permissible limit |  |
| --- | --- | --- | --- |
|  |  | Land/ Underground | Surface water courses |
| Total coliforms | MPN per 100 ml | - | <400 |
| E. Coli | MPN per 100 ml | <1000 | <200 |
| Free Chlorine Total Suspended Solids (TSS) Reactive Phosphorus | mg/l l mg/l mg/l | - 45 10 | 0.5 35 1 |
| Colour | - | Not objectionable |  |
| Temperature | 0C | 40 |  |
| pH | - | 5 – 9 |  |
| Chemical Oxygen Demand (COD) | mg/l | 120 |  |
| Biochemical Oxygen Demand (BOD5) | mg/l | 40 |  |
| Chloride | mg/l | 750 |  |
| Sulphate | mg/l | 750 |  |
| Sulphide | mg/l | 0.002 |  |
| Ammoniacal Nitrogen | mg/l | 1 |  |
| Nitrate as N | mg/l | 10 |  |
| Total Kjeldahl Nitrogen (TKN) | mg/l | 25 |  |
| Nitrite as N | mg/l | 1 |  |
| Aluminium | mg/l | 5 |  |
| Arsenic | mg/l | 0.1 |  |
| Beryllium | mg/l | 0.1 |  |
| Boron | mg/l | 0.75 |  |
| Cadmium | mg/l | 0.01 |  |
| Cobalt | mg/l | 0.05 |  |
| Copper | mg/l | 0.5 |  |
| Iron | mg/l | 2.0 |  |
| Lead | mg/l | 0.05 |  |
| Lithium | mg/l | 2.5 |  |
| Manganese | mg/l | 0.2 |  |
| Mercury | mg/l | 0.005 |  |
| Molybdenum | mg/l | 0.01 |  |
| Nickel | mg/l | 0.1 |  |
| Selenium | mg/l | 0.02 |  |
| Sodium | mg/l | 200 |  |
| Total Chromium | mg/l | 0.05 |  |
| Vanadium | mg/l | 0.1 |  |
| Zinc | mg/l | 2 |  |
| Oil & Grease | mg/l | 10 |  |
| Total Pesticides | mg/l | 0.025 |  |
| Total organic halides | mg/l | 1 |  |
| Cyanide (as CN -) or Free cyanide | mg/l | 0.1 |  |
| Phenols | mg/l | 0.5 |  |
| Detergents (as LAS*) | mg/l | 15 |  |

* Linear Alkylate Sulphonate

- THIRD SCHEDULE

(regulation 6)

Effluent discharge Standards

| Parameter | Unit | Maximum permissible limit |
| --- | --- | --- |
| Total coliforms | MPN per 100 ml | <400 |
| E. Coli | MPN per 100 ml | <200 |
| Free Chlorine Total Suspended Solids (TSS) Reactive Phosphorus | mg/l mg/l mg/l | 0.5 35 1 |
| Colour | - | Not objectionable |
| Temperature | 0C | 40 |
| pH | - | 5 – 9 |
| Chemical Oxygen Demand (COD) | mg/l | 120 |
| Biochemical Oxygen Demand (BOD5) | mg/l | 40 |
| Chloride | mg/l | 1500 |
| Sulphate | mg/l | 1500 |
| Sulphide | mg/l | 0.002 |
| Ammoniacal Nitrogen | mg/l | 1 |
| Nitrate as N | mg/l | 10 |
| Total Kjeldahl Nitrogen (TKN) | mg/l | 25 |
| Nitrite as N | mg/l | 1 |
| Aluminium | mg/l | 5 |
| Arsenic | mg/l | 0.1 |
| Beryllium | mg/l | 0.1 |
| Boron | mg/l | 0.75 |
| Cadmium | mg/l | 0.01 |
| Cobalt | mg/l | 0.05 |
| Copper | mg/l | 0.5 |
| Iron | mg/l | 2.0 |
| Lead | mg/l | 0.05 |
| Lithium | mg/l | 2.5 |
| Manganese | mg/l | 0.2 |
| Mercury | mg/l | 0.005 |
| Molybdenum | mg/l | 0.01 |
| Nickel | mg/l | 0.1 |
| Selenium | mg/l | 0.02 |
| Sodium | mg/l | 200 |
| Total Chromium | mg/l | 0.05 |
| Vanadium | mg/l | 0.1 |
| Zinc | mg/l | 2 |
| Oil & Grease | mg/l | 10 |
| Total Pesticides | mg/l | 0.025 |
| Total organic halides | mg/l | 1 |
| Cyanide (as CN -) | mg/l | 0.1 |
| Phenols | mg/l | 0.5 |
| Detergents (as LAS*) | mg/l | 15 |

* Linear Alkylate Sulphonate