S.I. 83 of 1995

ENVIRONMENT PROTECTION ACT, 1994

(Act 9 of. 1994)

Environment Protection (Standards)

Regulations, 1995

In exercise of the powers conferred by sections 6 and 40 of the Environment Protection Act, 1994, the Minister of Foreign Affairs, Planning and Environment hereby makes the following Regulations----

1. These Regulations may be cited as the Environment Protection (Standards) Regulations, 1995.

2. In these Regulations ---

“recipients system” means the part of the environment such as soil, water or rain or other medium which receives an effluent.

3. The Effluent Quality Standard specified in the Schedule is hereby prescribed as the Standard for the discharge of effluents to a recipient system from any industry,

operation or process.

SCHEDULE

1. In this Schedule

(a) Effluent Quality Standard means the maximum acceptable concentrations in an effluent as specified in paragraph 2.

(b) BOD5 means Biochemical Oxygen Demand, which is a measure of the organic matter content of a sample that is susceptible to oxidation by micro-organisms at an incubational temperature of 20oc over a period of five days.

(c) COD means Chemical Oxygen Demand, which s a measure of the oxygen equivalent of the organic matter content of a sample that is susceptible to oxidation by a strong chemical oxidant.

2. Effluent Quality Standard

| Parameters | Maximum concentration in milligram Per litre (mg/1) unless otherwise stated (except pH) |
| --- | --- |
| Temperature | 30oC measured at the point of Discharge |
| PH | 5.5 – 8.5 |
| Suspended Solids | 30 |
| BOD5 at 20oC | 30 |
| COD | 80 |
| Free Chlorine (as Cl₂) | 0.5 |
| Phosphorus (as PO4₃) | 5 |
| Nitrate (as NO₃) | 15 |
| Nitrite (As NO₂) | 1 |
| Phenols | 0.1 |
| Chromium (total) | 1.0 |
| Arsenic (total) | 0.1 |
| Mercury (total) | 0.05 |
| Cadmium (total) | 0.2 |
| Lead (total) | 0.9 |
| Copper (total) 1 |  |
| Zinc (total) 2 |  |
| Iron (total) 5.0 |  |
| Nickel (total) 1 |  |
| Aluminum (total) | 1 |
| Tin (total) | 0.1 |
| Manganese (total) | 2.0 |
| Oil and grease | 10 |
| Total coliforms | 500/100ml |
| Faecal coliforms | 100/100 ml |
| Faecal streptococcus | 100/100 ml |
| Salmonella | Must not be detectable in any 100 ml sample |
| Pesticides | In accordance with the law relating to pesticides which may be in force |

3. Sampling

For the purpose of this Standard sampling shall be carried out in accordance with the British Standards 6068 – Guidance on Sampling of Waste Waters: Section 6.10: 1993, unless otherwise specified by the Authority.

4. Reference Methods of Analysis

For the purpose of this standard, the Reference Methods of Analysis shall be based on the methods, as adopted by the Laboratory established or recognized under section 26 of the Act.

MADE this 10th day of August, 1995.