Air Pollution Control

-----------------------------------------------------------------------------------------------------

This design case models a sequence of three units for removing solid particles (Dust) and a volatile organic compound (Acetone) from an air stream with a volumetric flowrate of around 8,500 m3/h.

The majority of the large particles are removed using a cyclone (CY-101). A baghouse filter (BHF-101) is used to remove the remaining particles. Acetone is captured using a scrubber. Water was selected as the scrubbing liquid because acetone is highly soluble in water. A scrubber with a diameter of 1.8 m and a height of 5.58 m is required to remove 99.95% of acetone.

Note that the absorber can estimate removal efficiency if it is in Rating Mode and you specify its dimensions (diameter and height).