Incineration Example

-----------------------------------------------------------------------------------------------------

This example models an incinerator that is used to burn sludge. The assumed composition of the sludge is shown below:

Component                 wt %

Water                     75.0

Carbon                     5.5

Hydrogen                   4.0

Oxygen                     6.0

Chlorine                   1.4

Sulfur                     2.1

Ash                        6.0

The following assumptions are made in the material balance calculations:

1) All carbon in the feed streams is converted to carbon dioxide.

2) All sulfur is converted to sulfur dioxide.

3) Chlorine combines with hydrogen to yield hydrogen chloride.

4) The remaining hydrogen is oxidized to water.

5) All ash is released as ash in the bottom exit stream of the incinerator. A small fraction, which is determined by the value of "Ash To Flue Gas" that is displayed on the Oper. Conditions tab, ends up in the Flue Gas stream.

The air stream flowrate is adjusted by the program based on the stoichiometrically required amount and the Excess Oxygen %. If the sludge cannot be combusted autothermally, then, the user must specify a fuel component the flowrate of which is adjusted by the program if the "Adjust Fuel Flow for Complete Combustion of Waste" button is checked.