ß-Galactosidase Production

---------------------------------------------------------------------------------------------------------

This example deals with ß-galactosidase (b-Gal), an intracellular enzyme produced by Escherichia coli. This enzyme is normally produced by E. coli up to 1-2% of total cell protein under conditions of induction of the lac operon. Using genetic engineering, the level can go up to 20-25% of total protein. In this example, an easily attainable level of 10% is assumed.

b-Gal has found limited industrial applications until now. It is mainly used in the utilization of cheese whey. More specifically, immobilized reactors with b-Gal have been developed to convert lactose found in cheese whey to glucose and galactose, yielding a sweetened product which can be used as an additive to ice cream, egg-nog, yogurt, and other dairy products. Another application of b-Gal is in the treatment of milk products. A significant number of people are lactose intolerant and cannot digest milk or milk products. Production of lactose-free milk products (using b-Gal reactors) allows those people to digest them.

This example analyzes a plant that produces 11,500 kg of b-Gal per year in 134 batches.  Several files have been included with this example:

BGal-a:  This file represents the process at an early stage of plant design.  All equipment is in Design Mode, meaning that all equipment volumes and throughputs are calculated as opposed to being specified by the user.

BGal-b:  This file shows the plant after equipment sizes have been specified for key pieces of equipment.  In addition, some equipment is reused for multiple unit procedures.  The BGal-b file was used to produce the tables and graphs in the rest of this document.

BGal-c:  This file is the same as BGal-b, except that final product formulation and packaging unit procedures have been added.

PROCESS DESCRIPTION (file BGal-b)

Flowsheet Sections

The concept of flowsheet sections was introduced as part of release 3.0 to facilitate reporting of results for costing, economic evaluation, and raw material requirements of integrated processes. A flowsheet section is a group of process steps that have something in common. For instance, the ß-galactosidase flowsheet has been divided into three sections: 1) Fermentation, 2) Primary Recovery, and 3) Purification (see below). All the process step icons of the Fermentation section are displayed in black, while the icons of the Primary Recovery and Purification sections are displayed in blue and green, respectively. For information on how to specify flowsheet sections and edit their properties, please see Chapter 7.1.

Fermentation Section

Fermentation media are prepared in a stainless steel tank (V-101) and sterilized in a continuous sterilizer (ST-101).  A compressor (G-101) and an absolute air filter (AF-101) provide sterile air to the fermentor (FR-101).  Gaseous ammonia is used as a nitrogen source.

Primary Recovery Section

The first step of the primary recovery section is cell harvesting to reduce the volume of the broth and remove extracellular impurities; it is carried out by a membrane microfilter (MF-101).  Since ß–galactosidase is an intracellular product, the next step is cell disruption, performed in a high-pressure homogenizer (HG-101).  Storage vessels (V-101, 102, 103, and 107) are used at various stages of the process to act as surge capacity for certain unit procedures with restricted throughputs.  For instance, upstream of the P-7/HG-101 homogenizer is the P-15/V-101 storage vessel.  This vessel is necessary because the time required for homogenization in HG-101 is greater than the time required for microfiltration in MF-101.  Since these pieces of equipment do not have storage capacity within them, the difference in flow rates creates a necessity for intermediate storage equipment such as V-101.  Notice that some of these vessels are used multiple times within the process (V-101, V-102, and V-103 are each used for two different steps.)  After homogenization, a disk–stack centrifuge (DS-101) is used to remove most of the cell debris particles.  A dead-end polishing filter (DE-101) removes the remaining cell debris particles.  The resulting protein solution is concentrated by an ultrafilter (UF-101) and stored in V-103.

Purification Section

Next the product stream is purified by an ion exchange chromatography column (C-101). Then it is concentrated by a secondultrafiltration (UF-101) and polished by a gel filtration unit (C-102). A diafiltration unit (DF-101) exchanges the gel filtration buffer, and the protein solution is lyophilized in a freeze dryer (FDR-101).

OVERALL MATERIAL BALANCES

The table below provides information on raw material requirements for the entire flowsheet.  The quantities are displayed in kg/year, kg/batch, and kg/kg MP (main product). Around 86 kg of purified b-Gal is produced per batch. Note the large amounts of buffers and WFI that are required per batch.

*MP = “Main Product”, which is purified b-Gal in this case.

The above table was extracted from the Excel version of the Stream Report (File \ Export Reports to Excel...). The report was opened using Excel, edited, and pasted into this document.

PROCESS SCHEDULING

The figure below is an Equipment Utilization chart generated by Pro-Designer. It shows the execution of the various process steps as a function of time for three consecutive batches. The overall batch time is approximately 150.7 hours. The minimum effective batch time (minimum time interval between consecutive batches) calculated by Pro-Designer is 56.25 hours.  This is determined by the occupancy of the scheduling bottleneck equipment, C101. The maximum number of batches per year is 139 (assuming an annual operating time of 7920 h and a plant batch time equal to the minimum effective batch time). The actual number of batches in this example is 134. On an annual basis, the plant produces 11,500 kg of purified b-Gal.

In addition to the above Equipment Utilization Chart, SuperPro Designer generates and operations Gantt charts that provides a detailed visual display of the execution of the various operations in the context of their procedures.

Charts such as the ones above allow for easy visualization of the time (scheduling) bottlenecks.  Along with the Throughput Analysis Report, these charts greatly simplify the debottlenecking of complex processes.  Please see the User’s Guide for more information on performing throughput analyses using these tools.

COST ANALYSIS AND ECONOMIC EVALUATION

The key results of cost analysis for a plant producing 11,538 kg of b-Gal per year follow.  This analysis assumes that a new facility will be built for this process, and the project lifetime is 15 years.

| TOTAL CAPITAL INVESTMENT | 63,386,000 | $ |
| --- | --- | --- |
| OPERATING COST | 34,922,000 | $/year |
| PRODUCTION RATE | 11,538 | kg/year of B-Gal |
| UNIT PRODUCTION COST | 3,027 | $/kg of B-Gal |
| TOTAL REVENUES | 63,462,000 | $/year |
| GROSS MARGIN | 44.97 | % |
| RETURN ON INVESTMENT | 35.84 | % |
| PAYBACK TIME | 2.79 | years |
| IRR AFTER TAXES | 24.14 | % |
| NPV (at 7.0 % interest) | 79,127,000 | $ |

The above table is copied from the Economic Evaluation Report generated by SuperPro Designer.  This table gives an overall summary of the economics of the process.  The information above (as well as additional information) is also displayed on the Executive Summary dialog (View \ Executive Summary). Much more detailed information is available in additional tables in the Economic Evaluation Report and the Itemized Cost Report.  One such table is the Annual Operating Cost summary below.  This breaks down the costs into general categories.  From this table, it is clear that raw material and equipment-dependent costs account for the majority of the annual expense for this process (note – the equipment costs include depreciation and maintenance.)  Consumables (especially the chromatography resins) and labor are also significant expenses.

ANNUAL OPERATING COST - SUMMARY  (1999 prices)

| Cost Item | $/Year | % |
| --- | --- | --- |
| Raw Materials | 12,706,000 | 36.38 |
| Labor-Dependent | 4,024,000 | 11.52 |
| Equipment-Dependent | 11,097,000 | 31.77 |
| Laboratory/QC/QA | 925,000 | 2.65 |
| Consumables | 5,590,000 | 16.01 |
| Waste Treatment/Disposal | 308,000 | 0.88 |
| Utilities | 274,000 | 0.79 |
| TOTAL | 34,924,000 | 100 |

Looking more closely at the raw material expenses (see table below), it is clear that the WFI and buffers are the largest material expenses for this process.  Other tables in the Economic Evaluation Report show the cost of each piece of equipment, as well as the costs for installation, piping, instrumentation, etc.  The user should look at each of these expenses to ensure they are accurate for their particular application.  Additional tables in the Economic Evaluation Report include: breakdowns of costs for labor, consumables, waste treatment, and utilities, profitability and cash flow analyses, and loan information.

| Raw | Unit Cost | Annual Amount | Cost |  |
| --- | --- | --- | --- | --- |
| Material | ($/kg) | (kg) | ($/yr) | % |
| Process Water | 0.01 | 8,576,000 | 85,760 | 0.67 |
| Glucose | 1 | 823,162 | 823,162 | 6.48 |
| Salts | 1.6 | 137,484 | 219,974 | 1.73 |
| Ammonia | 0.5 | 52,930 | 26,465 | 0.21 |
| WFI | 0.1 | 31,972,978 | 3,197,298 | 25.16 |
| Tris Buffer | 0.15 | 25,258,405 | 3,788,761 | 29.82 |
| NaCl (0.1 M) | 0.13 | 20,692,755 | 2,690,058 | 21.17 |
| NaOH (0.5 M) | 0.12 | 12,761,902 | 1,531,428 | 12.05 |
| NaCI (0.5 M) | 0.19 | 1,806,727 | 343,278 | 2.7 |
| TOTAL |  | 102,082,343 | 12,706,184 | 100 |

In addition to the tables in the Economic Evaluation Report, the Itemized Cost Report displays breakdowns of various costs for each section of the process.  For instance, the table below provides information on the contribution of the Fermentation, Primary Recovery, and Purification sections to the process costs. The Purification section contributes 74% of the total operating cost. Overall, around 88% of the total operating cost is associated with the Primary Recovery and Purification sections. This is quite common for high-value biological products.

The figure below displays the unit cost (in $/kg of b-Gal) as a function of production rate.  For this example, all equipment items were set to “Design Mode” so that all equipment would be scaled relative to the required throughput.  The plant throughput was adjusted by selecting Tasks / Adjust Plant Throughput… from the main menu. For more information on plant throughput adjustment, please see section 2.2.19 in Chapter 2. Notice that the unit cost is very sensitive to production rate at low annual throughputs (less than 10,000 kg/yr). At higher annual throughputs, the cost is relatively insensitive to production rate.

| Cost Item | Fermentatio | Primary Rec | Purificatio | Subtotal | % |
| --- | --- | --- | --- | --- | --- |
|  | $/year | $/year | $/year | $/year |  |
| Raw Materials | 1,155,361 | 235,313 | 11,315,510 | 12,706,184 | 36.38 |
| Equipment | 1,972,330 | 2,286,242 | 6,838,385 | 11,096,956 | 31.77 |
| Labor | 758,781 | 1,466,384 | 1,799,086 | 4,024,251 | 11.52 |
| Consumables | - | 382,265 | 5,207,657 | 5,589,922 | 16.01 |
| Lab/QC/QA | 234,500 | 321,600 | 368,500 | 924,600 | 2.65 |
| Waste Trt/Dsp | - | 83,563 | 223,956 | 307,518 | 0.88 |
| Utilities | 194,888 | 71,498 | 7,865 | 274,251 | 0.79 |
| Subtotal | 4,315,860 | 4,846,864 | 25,760,958 | 34,923,682 | 100 |
| Contribution (%) | 12.36 | 13.88 | 73.76 | 100 |  |

THROUGHPUT ANALYSIS

The Throughput Analysis Report (THR) presents information on equipment capacity utilization and the potential for throughput increase without installation of additional equipment. It is particularly useful in analyzing existing manufacturing facilities in an effort to increase annual production rate.  This report can be created by selecting Tasks / Generate Throughput Analysis Report (THR)... and displayed by selecting View / Throughput Analysis Report.  For detailed information on definitions and calculation methods, please consult the Help Facility.

The tables that follow are from the Throughput Analysis of the Bgal-b design case, in which the equipment calculation mode for the key process steps has been switched to Rating. Overall, C-101 has the highest combined utilization (81.27%) and therefore is the limiting step of the entire flowsheet. In other words, if we try to increase batch throughput (material processed per batch) without installing extra equipment, C-101 will become the first throughput bottleneck. The current batch size is 86 kg of purified b-galactosidase.  Without installing additional equipment, the maximum possible batch throughput is 106 kg of b-galactosidase, which corresponds to 100% utilization of C-101.

Additional tables in the Throughput Analysis Report point out the limiting steps of each individual section of the process (Fermentation, Primary Recovery, and Purification).

| Equipment | Procedure | Capacity | Equipment | Combined |
| --- | --- | --- | --- | --- |
| Tag | Name | Utilization | Uptime | Utilization |
|  |  | (%) | (%) | (%) |
| V-101 | P-1 | 95.78 | 37.42 | 35.84 |
|  | P-15 | 72.99 | 37.42 | 27.31 |
| HG-101 | P-7 | 100 | 18.05 | 18.05 |
| DE-101 | P-9 | 100 | 27.47 | 27.47 |
| UF-101 | P-10 | 100 | 25.63 | 25.63 |
|  | P-13 | 100 | 25.63 | 25.63 |
| C-102 | P-11 | 100 | 69.52 | 69.52 |
| DF-101 | P-12 | 100 | 70.91 | 70.91 |
| MF-101 | P-14 | 100 | 18.12 | 18.12 |
| C-101 | P-19 | 84.16 | 96.57 | 81.27 |
| FR-101 | P-20 | 95.78 | 45.48 | 43.56 |
| DS-101 | P-21 | 100 | 14.53 | 14.53 |
| V-102 | P-6 | 97.21 | 36.34 | 35.32 |
|  | P-8 | 70.29 | 36.34 | 25.54 |
| V-107 | P-18 | 64.29 | 54.15 | 34.81 |
| V-103 | P-16 | 72.99 | 36.4 | 26.57 |
|  | P-17 | 35.14 | 36.4 | 12.79 |
| Limiting Equipment / Procedure |  |  | C-101 / P-19 |  |

Another table in the Throughput Analysis Report is the Summary of Throughput Increase Options (below).  This table shows the potential maximum throughput (PMT) for each unit procedure.  The PMT is calculated in three ways, with different assumptions associated with each.  In all three cases, the PMT is based on 100% Capacity Utilization.  However, the Equipment Uptimes for the three cases differ:

- In the Conservative case:  For a procedure which has a cycle time that is not proportional to throughput, the Equipment Uptime is kept the same as its current uptime.  In cases where the procedure cycle time is proportional to throughput (such as filtrations), the Equipment Uptime is set at 100%.

- In the Realistic case:  For a procedure which has a cycle time that is not proportional to throughput, the Equipment Uptime is set to correspond to the highest number of complete cycles that can be performed in that piece of equipment within the time window of the effective plant batch time.  In this example, the unit procedure P-20 in FR-101 has an equipment uptime of 45.48% (refer back to the Equipment Capacity Utilization and Uptime table).  Therefore, up to 2 full cycles could be run in FR-101 without this piece of equipment becoming a time bottleneck.  In cases where the procedure cycle time is proportional to throughput, the Equipment Uptime is set at 100%.

- In the Theoretical case:  All equipment is assumed to be at 100% Capacity Utilization all the time (100% Equipment Uptime).  This would require mixing of partial lots of product, which may not be an option in many processes.

| SUMMARY OF THROUGHPUT INCREASE OPTIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| (Throughput in kg MP/batch) |  |  |  |  |
| Equipment | Procedure | POTENTIAL MAXIMUM THROUGHPUT |  |  |
| Tag | Name | ## Conservative | Realistic | Theoretical |
| V-101 | P-1 | 191.94 | 191.94 | 191.94 |
|  | P-15 | 251.84 | 251.84 | 251.84 |
| DE-101 | P-9 | 313.47 | 313.47 | 313.47 |
| UF-101 | P-10 | 336.01 | 336.01 | 336.01 |
|  | P-13 | 336.01 | 336.01 | 336.01 |
| C-102 | P-11 | 86.11 | 86.11 | 123.87 |
| MF-101 | P-14 | 445.36 | 445.36 | 445.36 |
| C-101 | P-19 | 102.32 | 102.32 | 105.96 |
| FR-101 | P-20 | 89.91 | 179.81 | 197.67 |
| V-102 | P-6 | 243.33 | 243.33 | 243.33 |
|  | P-8 | 336.55 | 336.55 | 336.55 |
| V-107 | P-18 | 133.93 | 133.93 | 247.34 |
| V-103 | P-16 | 257.09 | 257.09 | 257.09 |
|  | P-17 | 533.98 | 533.98 | 533.98 |
| Limiting Equipment |  | C-102 | C-102 | C-101 |
| Limiting Procedure |  | P-11 | P-11 | P-19 |

Please see the User’s Guide and/or consult the Help Facility for more information on throughput analysis and debottlenecking.

PRODUCT FORMULATION, PACKAGING, AND DISTRIBUTION

The BGal-c design case file displays a modified version of the base case flowsheet that includes unit procedure models for product formulation, packaging, and distribution. Most of these models are rather simple and their primary objective is to capture the cost associated with such process steps.

Product formulation and packaging operations often involve formation and use of discrete entities, such as tablets, bottles, boxes, etc. The flow of such entities is represented by discrete streams, which by default are displayed in blue color. For more information on discrete streams and entities, please consult the Help Facility and the User’s Guide.

To familiarize yourself with the formulation and packaging unit procedure models and the concepts of discrete streams and entities, please open the BGal-c design case and visit the simulation data dialog windows of these objects. You can do that by right clicking on the various packaging unit procedure icons and their corresponding streams and selecting Operation (or Simulation) Data. Notice the different interface of discrete streams, which display the flow of discrete entities as well as the equivalent bulk flow (based on the bulk ingredients that compose the discrete entities).