Monoclonal Antibody Production

---------------------------------------------------------------------------------------------------------

This example analyzes the production of a therapeutic monoclonal antibody using animal cell culture. Approximately 6.35 kg of purified product is produced per year in 47 batches. This scale is typical of therapeutic monoclonal antibody production.

Several SuperPro flowsheet files have been included with this example:

Mab_4_0a:   This flowsheet shows the base case for the plant.  The Mab_4_a file was used to produce the tables and graphs in the rest of this document.

Mab_4_0b:   This flowsheet includes final product packaging and transportation unit procedures.

Below is a brief description of the basic features of the monoclonal antibody process (file Mab_4_0a).  All files for this example can be found in the Examples\MammCell subdirectory.

# Upstream Processing

First, a serum-free low-protein media powder is dissolved in WFI in a stainless steel tank (V-101).  Then the solution is sterilized using a 0.1 m dead-end polishing filter (DE-101). The concentration of media powder in the feed solution is 10 g/liter. A stirred-tank bioreactor (V-102) is used to grow the cells, which produce the therapeutic antibody “IgG”. The bioreactor operates in fed-batch mode. A cycle time of 145.1 hours (132 hours for fermentation and 13.1 hours for material transfers and cleaning) was assumed for the bioreactor. The volume of broth generated per bioreactor batch is approximately 2,200 liters, which contains roughly 220 g of product (the product titer is 100 g/L). The total volume of the bioreactor vessel is 3,000 liters. For the sake of simplicity, the train of seed bioreactors was ignored in this example.

Downstream Processing

The generated biomass and other suspended compounds are removed using a 0.65 m membrane diafilter (DF-101). During this step, roughly 5% of the IgG denatures, and some additional IgG is lost in the Cell Waste stream.  As a result, the product yield during this step is around 89%. The clarified solution is then concentrated fifteen fold using a 50,000 MW cut-off ultrafilter (UF-101). The product yield of this step is 95%. The bulk of the contaminant proteins are removed using a “protein A” affinity chromatography column (C-101). The protein A elution buffer is then exchanged with phosphate buffer using a 50,000 MW cut-off diafilter (DF-102). The product recovery yield of this step is 95%. The purification proceeds using a cation exchange chromatography column (C-102). Next, ammonium sulfate is added to increase the ionic strength of the solution and prepare it for the hydrophobic interaction chromatography (HIC) step (C-103) that follows. The purified product solution is diafiltered with PBS buffer (DF-103) and is mixed with glycerol to a final concentration of 100 g of glycerol per liter (glycerol increases the shelf-life of the product). It is then sent through a final dead-end filter (DE-102).  The concentration of IgG in the final product stream is 0.38%. The overall product yield for the downstream processing (Recovery and Purification sections) is 62%.

# Flowsheet Sections

A flowsheet section is a group of process steps that have something in common. This flowsheet has been divided into three sections: 1) Bioreaction, 2) Recovery, and 3) Purification. All the process step icons of the Bioreaction section are displayed in black, while the icons of the Recovery, Purification, and Packaging sections are displayed in green, blue and purple, respectively. For information on how to specify flowsheet sections and edit their properties, please consult the Help Facility and the User’s guide.

The user is advised at this point to visit the various process steps and check their input and output data. This can be done by right-clicking (clicking once on the right mouse button) on a process step icon, selecting Operation Data..., and choosing the desired operation. The stream flowrates and compositions can be checked by double-clicking (with the left mouse button) on any stream.

NOTE - The flowsheet described in this example was developed using data that are available in the literature (see References). Specifically, the recovery train was put together using the MAbAssistant Expert System from Pharmacia Biotech. It is likely that certain assumptions used to generate this example may not be realistic. The primary purpose of this example is to demonstrate the use of Pro-Designer for modeling and evaluating animal cell processes. Do not use this example as the basis for designing actual processes.

RAW MATERIAL REQUIREMENTS

The table below displays the raw material requirements in kg/batch, kg/year, and kg/kg MP (main product). This plant produces approximately 135 g of therapeutic IgG per batch. Note the large amounts of WFI and PBS that are required per batch. The table below was extracted from the Excel version of the Stream Report.

| Raw Material | kg/Year | kg/Batch | kg/kg MP |
| --- | --- | --- | --- |
| Media | 1,023 | 21.8 | 161.1 |
| WFI | 156,721 | 3,334.5 | 24,689.0 |
| Air | 28,797 | 612.7 | 4,536.5 |
| PBS | 136,129 | 2,896.4 | 21,445.1 |
| Sodium Chloride | 1,159 | 24.7 | 182.7 |
| TRIS HCl | 32 | 0.7 | 5.1 |
| Prot-A Reg Buff | 20,781 | 442.2 | 3,273.8 |
| Na2HPO4 | 221 | 4.7 | 34.8 |
| Amm. Sulfate | 3,041 | 64.7 | 479.0 |
| Sodium Hydroxid | 232 | 4.9 | 36.6 |
| Glycerin | 161 | 3.4 | 25.3 |
| NaOH (0.5 M) | 4,476 | 95.2 | 705.2 |
| NaH2PO4 | 41 | 0.9 | 6.4 |
| NaCI (1 M) | 6,092 | 129.6 | 959.7 |
| Flowsheet Total | 358,906 | 7,636.3 | 56,540.3 |

PROCESS SCHEDULING

The figure below is an Equipment Utilization chart generated by Pro-Designer. It shows the execution of the various process steps as a function of time for two consecutive batches. A new batch is initiated every seven days. On an annual basis, the plant processes 47 batches and produces 6.35 kg of purified IgG. The time required to run one batch from start to finish is 180.25 hours.

Equipment Utilization Chart - Two consecutive Batches.

In addition to the Equipment Utilization Chart, SuperPro Designer creates Gantt charts for the various operations and their unit procedures. In the Operations Gantt chart that follows, the tan bar at the top represents the time required for one full batch. The duration of each section of the flowsheet (Bioreaction, Recovery, and Purification) is displayed with green bars. The dark blue bars represent the time required for each unit procedure, and the light blue bars represent individual operations. Both the Operation and Equipment Gantt charts are highly customizable. To modify the bar properties (such as their colors, labels for duration, etc.), right-click on the bar area and choose “Styles” from the drop-down list. Then choose a bar type to edit. From these dialogs, the user can also specify which bars will be included on the chart (for instance, only the bars for individual operations could be shown, if desired.)

Charts such as the Operations Gantt chart and the Equipment Utilization Chart can be powerful tools for throughput analysis, debottlenecking, planning purposes, and technology transfer.  The manual which comes with this software contains much more information on throughput analysis and debottlenecking (including a step-by-step demonstration of debottlenecking a synthetic pharmaceutical process).

Above:  The single-batch Operations Gantt Chart. This chart was copied from Pro-Designer and pasted into this document using OLE-2 technology.

ECONOMIC EVALUATION

The key economic evaluation figures for a plant dedicated to production of 6.35 kg of IgG per year are displayed in the table below.  This table was extracted from the Excel version of the Economic Evaluation Report.  This information (along with additional data) also appears in the Executive Summary dialog.  This analysis assumes that a new facility will be built for this process, and the plant lifetime is 15 years.  The revenues are based on a selling price of $2,500,000/kg of IgG.

| TOTAL CAPITAL INVESTMENT | 15,400,000 | $ |
| --- | --- | --- |
| OPERATING COST | 4,707,000 | $/year |
| PRODUCTION RATE | 6.348 | kg/year of IgG (in S-102) |
| UNIT PRODUCTION COST | 741,475 | $/kg of IgG (in S-102) |
| TOTAL REVENUES | 15,869,000 | $/year |
| GROSS MARGIN | 70.34 | % |
| RETURN ON INVESTMENT | 52.47 | % |
| PAYBACK TIME | 1.91 | years |
| IRR AFTER TAXES | 39.30% | % |
| NPV (at 7.0 % interest) | 38,828,000 | $ |

The next table below gives more information on the annual operating cost.  It was extracted from the Itemized Cost Report (ICR) of this example.  Much more detailed breakdowns of various costs (raw materials, equipment, labor, etc) are also available in the Economic Impact Report and Itemized Cost Report.  To save space, these additional tables are not reproduced below.

| Cost Item | Bioreaction | Recovery Se | Purificatio | Subtotal | % |
| --- | --- | --- | --- | --- | --- |
|  | $/year | $/year | $/year | $/year |  |
| Raw Materials | 593,074 | 28,944 | 57,384 | 679,402 | 14.43 |
| Equipment | 1,132,243 | 496,058 | 1,032,123 | 2,660,425 | 56.52 |
| Labor | 328,862 | 64,811 | 105,735 | 499,408 | 10.61 |
| Consumables | 5,139 | 403,641 | 212,436 | 621,216 | 13.2 |
| Lab/QC/QA | 49,329 | 9,722 | 15,860 | 74,911 | 1.59 |
| Waste Trt/Dsp | - | 93,617 | 70,606 | 164,223 | 3.49 |
| Utilities | 6,399 | 537 | 199 | 7,136 | 0.15 |
| Subtotal | 2,115,047 | 1,097,331 | 1,494,343 | 4,706,722 | 100 |
| Contribution (%) | 44.94 | 23.31 | 31.75 | 100 |  |

As you can see from the right-hand column of the table above, equipment-dependent costs are the largest expense, followed by raw materials and consumables.  This is common for biopharmaceutical plants that produce small amounts of high value products.  The equipment-dependent cost was calculated based on depreciation, maintenance and miscellaneous costs. The labor cost was calculated from a base labor rate per operator hour plus adjustments for fringe benefits, administration, etc.  This table also provides information on the contribution of each flowsheet section (Bioreaction, Recovery, and Purification) to various types of cost.

THROUGHPUT ANALYSIS

The Throughput Analysis Report (THR) presents information on equipment capacity utilization and the potential for throughput increase without installation of additional equipment. It is particularly useful in analyzing existing manufacturing facilities in an effort to increase production rate.  This report can be created by selecting Tasks / Generate Throughput Analysis Report (THR)... and displayed by selecting View / Throughput Analysis Report.  For detailed information on definitions and calculation methods, please consult the Help Facility and the User’s Guide.

For this particular example, it can easily be seen that V-102 is the scheduling bottleneck (refer to the Equipment Utilization Chart shown earlier).  You can also determine this from the Equipment Uptime column of the table below.  In addition, you can use this table to determine the Capacity (size) Utilization of each unit procedure.  The Combined Utilization column of this table is simply the product of the Equipment Uptime and the Capacity Utilization.  Since V-102 has the highest Combined Utilization of these unit procedures, it is the limiting step of the entire flowsheet. In other words, if we try to increase batch throughput (material processed per batch) without installing extra equipment, V-102 will become the first throughput bottleneck. The Equipment Capacity Utilization and Uptime table can be very useful when attempting to debottleneck complex processes (especially those that share equipment between multiple unit procedures).

Another table in the Throughput Analysis Report is the Summary of Throughput Increase Options Table, which shows the potential maximum throughput (PMT) for each unit procedure.  The PMT is calculated in three ways, with different assumptions associated with each.  For more information on throughput analysis, please refer to the Help Facility and the manual.

| Equipment | Procedure | Capacity | Equipment | Combined |
| --- | --- | --- | --- | --- |
| Tag | Name | Utilization | Uptime | Utilization |
|  |  | (%) | (%) | (%) |
| V-102 | P-5 | 91.11 | 86.37 | 78.70 |
| DF-101 | P-7 | 100.00 | 3.33 | 3.33 |
| V-103 | P-8 | 85.04 | 2.62 | 2.23 |
| UF-101 | P-9 | 100.00 | 2.42 | 2.42 |
| C-101 | P-10 | 50.04 | 9.32 | 4.66 |
| DF-102 | P-11 | 100.00 | 2.60 | 2.60 |
| C-102 | P-12 | 52.93 | 7.65 | 4.05 |
| V-104 | P-14 | 20.55 | 3.84 | 0.79 |
| C-103 | P-15 | 47.64 | 8.00 | 3.81 |
| DF-103 | P-17 | 100.00 | 2.36 | 2.36 |
| DE-102 | P-19 | 100.00 | 2.38 | 2.38 |
| Limiting Equipment / Procedure |  |  |  | V-102 / P-5 |

Above: the Equipment Capacity Utilization and Uptime table.

PRODUCT FORMULATION AND PACKAGING

At this point, please open the second file of this example (Mab_4_0b) and visit the product packaging and distribution process steps at the end of the production line. This group of unit procedure models can be used to represent and account for the cost of product formulation, packaging, and distribution. Unit procedure models are available for packaging of liquid as well as solid products. Transportation unit procedures (e.g., truck, train, plane, etc.) can be used to account for the cost of product distribution. Most of these models are rather simple, and their primary objective is to capture the cost associated with such procedures.

Product formulation and packaging operations often involve formation and use of discrete entities, such as tablets, bottles, boxes, etc. The flow of such entities is represented by discrete streams, which by default are displayed in blue. For more information on discrete streams and entities, please refer to Chapter 4 of the manual.

To familiarize yourself with the packaging and transportation unit procedure models and the concepts of discrete streams and entities, please open the Mab_4_0b design case and visit the simulation data dialog windows of these objects. You can do that by right-clicking on the various packaging unit procedure icons and their corresponding streams, and choosing either Operation Data or Simulation Data. Notice the different interface of discrete streams, which display the flow of discrete entities as well as the equivalent bulk flow (based on the bulk ingredients that compose the discrete entities).

REFERENCES

Monoclonal Antibody Purification Handbook (including MAbAssistant), Pharmacia, 1993.

Atkinson B. and F. Mavituna (1991), "Biochemical Engineering and Biotechnology Handbook", 2nd edition, Chapter 8, Stockton Press (MacMillan Publishers).

Smith G.L. (1994), "Large Scale Animal Cell Culture", in Bioprocess Engineering, edited by B.K. Lydersen, N.A. D'Elia, and K.L. Nelson, Wiley Interscience.