Integrated Cheese Plant

---------------------------------------------------------------------------------------------------------

This example deals with an integrated milk processing plant that produces cheese, butter, whey protein concentrate (WPC), and food-grade ethanol. The development of the example was based on data that is available in the literature (References 1 and 2).

The plant operates around the clock for 330 days a year.  On a daily basis it processes 2,000 metric tons of milk (83,333 kg/h) and produces 214 tons of cheese, 9 tons of butter, 211 tons of WPC, and 33.5 tons of 95% (by mass) ethanol.

The concept of flowsheet sections was introduced as part of SuperPro release 3.0 to facilitate reporting of results for costing, economic evaluation, and raw material requirements of integrated processes. A flowsheet section is a group of process steps that have something in common. The plant in this example consists of four sections: Cheese-Making, Butter-Making, WPC-Making, and Ethanol-Making. The process step icons of the Cheese, Butter, WPC, and Ethanol sections are displayed in black, blue, green, and dark red, respectively. For information on how to specify flowsheet sections and edit their properties, please see Chapter 7.1 of the manual or consult the Help Facility (search for “sections”).

PROCESS DESCRIPTION

Cheese Production

Milk is pumped from tankers into any of six identical 227 m3 (60,000 gal) silos (V-112), which are each mounted on load cells to monitor the level of milk at any given time.  (Note – these six silos are represented by one icon on the flowsheet. Other sets of identical equipment items, operating in parallel, are represented by single icons as well.  To view the number of equipment items represented by an individual icon, right-click on the icon and select “Equipment Data”.  The temperature in the silos is maintained at around 4 oC. Milk from the silos flows through a pasteurizer (HX-101), which is a heat exchanger operating at around 68 oC.  The milk then flows through a cooler (HX-102) to reduce its temperature at the inlet to the cheese vats to 30 oC.  Lactic-acid starter culture and renin are added in small proportions to the contents of the vats (V-102). Next, the curd is solidified for two hours in the Cheddar Tower (V-104).  Then the aqueous whey stream (S-109), is removed (CSP-101).  The whey stream is composed of unconsumed proteins, fat, lactose, mineral salts, and water. Next, salts are added (MX-102) to stream S-110 and the cheese is formed into blocks (V-103).  After block formation, the cheese is sealed (TB-101), cooled and stored to mature.

Butter Production

Using three centrifugal separators (DS-101), the whey stream (S-101) is separated into a fat-rich cream (S-127) and a dilute aqueous stream (S-111) containing mainly proteins, salts, and lactose. The cream is pasteurized as before (HX-103 and HX-104) and used as the feed for making Grade B butter, which is the highest possible grade for whey butter. The butter is packaged in cartons (FR-101), which contain 30 kg of material. In addition, the butter whey stream (S-125) is combined with the dilute aqueous stream (S-111) from the centrifugal separators and is sent to the WPC section.

# Whey Protein Concentrate (WPC) Production

The dilute aqueous streams (S-125 and S-111) from the Butter-Making section contain two other potentially valuable components, albeit in low concentrations: whey proteins and lactose. The whey proteins are concentrated using crossflow ultrafilters (UF-101), and then spray-dried (SDR-101) to produce solid WPC containing 50-75 % protein. The dried WPC is stored in silos (SL-101) and packaged in bags containing 22.5 kg (50 lbs) of material.

# Ethanol Production

The liquid permeate from the ultrafilters (stream S-137) is treated using reverse osmosis units (RO-101).  The concentrate stream from these units retains lactose and most of the mineral salts. The permeate stream (S-138) goes through another reverse osmosis system (RO-102) to remove residual minerals. The result is pure demineralized water, which is recycled as CIP sanitizing and rinse water. Approximately 33.5% of the water contained in the original milk is recycled this way. The concentrated lactose solution from the first RO system (stream S-115) is fed to any of three fermentors (V-101), where a proprietary strain of yeast is used to produce ethanol. The resulting “beer” solution, which is 4% to 5% alcohol (by mass) at this point, then flows into one of two beer wells (V-110).  Next, the beer solution is sent to a centrifugal separator (DS-102), which recovers and recycles the yeast.  The biomass-free permeate (stream S-140) is then pre-heated through a heat exchanger (HX-106) before distillation. Heat-exchange integration is used to warm the incoming solution (stream S-140) using the bottoms stream from the distillation column (stream S-143).  Next, the dilute ethanol solution is concentrated by distillation (C-101) to spirit-grade levels. The distillation process step is a typical three-stage system employing thermal vapor recompression to recover steam and using glycol to break the azeotrope in the first stage. This boosts the alcohol from 180 to 198 proof.

MATERIAL BALANCES

The table below provides information on raw material requirements for the entire flowsheet.  The quantities are displayed in kg/year, kg/hour, and kg/MP Entity (MP Entity = Main Product Entity = 18 kg cheese block).

| Raw Material | kg/Year | kg/Hour | kg/MP Entity |
| --- | --- | --- | --- |
| Starter | 6,599,974 | 833.3 | 1.681 |
| Renin | 659,997 | 83.3 | 0.168 |
| Salts | 1,412,806 | 178.4 | 0.360 |
| milk | 659,997,360 | 83,333.0 | 168.109 |
| Air | 123,227,008 | 15,559.0 | 31.387 |
| Flowsheet Total | 791,897,144 | 99,987.0 | 201.706 |

The above table was extracted from the Excel version of the Stream Report (File / Export Reports to Excel...). The report was opened using Excel, edited, and pasted into this document.

The following table is from the Excel version of the Environmental Impact Report.  It provides information on the amount of each entering component, and displays the amount of each that ends up in waste streams. For this process, the utilization of lactose is around 91%. In terms of water, around 61% of the incoming water (contained in milk) ends up in waste streams. The rest is recovered as demineralized water (around 33.5% of the total) or remains in the final products (as water content of cheese and butter). Overall, the plant is quite effective from an environmental impact point of view.

|  | TOTAL | OUT AS |
| --- | --- | --- |
| Component Name | IN | LIQUID WASTE |
|  | (kg/h) | (kg/h) |
| Anions | 333.3 | 311.2 |
| Casein | 2,250.0 | 0.0 |
| Cations | 250.0 | 233.4 |
| Ethyl Alcohol | 0.0 | 320.2 |
| Fats | 3,250.0 | 0.0 |
| Lactose | 4,083.3 | 381.2 |
| Nitrogen | 11,935.6 | 0.0 |
| Oxygen | 3,623.4 | 0.0 |
| Paper | 3.9 | 0.0 |
| Plastic | 12.3 | 0.0 |
| Renin | 83.3 | 0.0 |
| Salts | 178.4 | 0.0 |
| Starter | 833.3 | 0.0 |
| Water | 72,666.4 | 44,335.5 |
| Whey | 500.0 | 0.0 |
| Yeast | 0.0 | 343.1 |
| Plant Totals | 100,003 | 45,925 |

COST ANALYSIS AND ECONOMIC EVALUATION

The essential results of cost analysis for the entire flowsheet are shown below. The table below gives an overview of the total economic impact of the plant, including the total capital investment, yearly revenues, and rate of return.  This data was exported from the Economic Evaluation Report.  This information (along with additional data) also appears in the Executive Summary dialog.

| TOTAL CAPITAL INVESTMENT | 115,855,000 | $ |  |
| --- | --- | --- | --- |
| OPERATING COST | 173,353,000 | $/year |  |
| PRODUCTION RATE | 3,925,999 | Entities/year in Cheese |  |  |
| UNIT PRODUCTION COST | 44.16 | $/Entity in Cheese |  |  |
| TOTAL REVENUES | 266,443,000 | $/year |  |
| GROSS MARGIN | 34.94 | % |  |
| RETURN ON INVESTMENT | 56.18 | % |  |
| PAYBACK TIME | 1.78 | years |  |
| IRR AFTER TAXES | 46% |  |  |
| NPV (at 7.0 % interest) | 358,258,000 | $ |  |

Entity = Cheese Block (18 kg)

To calculate the revenues for this process, the following prices were assumed for the product and by-products of the plant: cheese is sold for $2.5/kg ($45/block), butter is sold for $2/kg ($80/carton), WPC is sold for $0.89/kg ($20/bag) and ethanol is sold for $0.9/kg.  The table below displays the total annual revenue generated by these products.

| REVENUE FROM ($/year) |  |
| --- | --- |
| Cheese | 176,670,000 |
| WPC | 4,408,000 |
| Ethanol | 9,401,000 |
| Butter Cartons | 75,964,000 |

The next table below gives a summary of the annual operating costs.  As can be seen from this table, raw materials contribute 82% of the total operating cost, followed by equipment-dependent costs (10.5%), labor (1.85%), and utilities (3.07%). The unit cost of milk was assumed to be $0.2/kg ($0.8/gal). The milk expense represents roughly 94% of the total raw material cost (see the second table below).  The equipment-dependent costs for this process are based on depreciation, maintenance, and miscellaneous equipment expenses. Labor costs were based on the sum of the labor requirements for each unit procedure multiplied by a fixed labor rate (which was based on a basic labor rate, plus adjustments for fringe benefits, administration, etc). Utility costs were based on the type of heat transfer agent used, and the heating/cooling requirements. The table below also provides information on the contribution of each flowsheet section to the total operating cost. As can be seen, the Cheese-making section contributes around 83.4% of the total cost, followed by Ethanol-making (8.54%), WPC-making (5.6%), and Butter-making (2.46%).

| Cost Item | Cheese Sect | Butter Sect | WPC Section | Ethanol Sec | Subtotal | % |
| --- | --- | --- | --- | --- | --- | --- |
|  | $/year | $/year | $/year | $/year | $/year |  |
| Raw Materials | 140,782,077 | 1,266,068 | 44,079 | 0 | 142,092,225 | 81.97 |
| Equipment | 2,115,394 | 1,849,382 | 5,735,247 | 8,524,209 | 18,224,234 | 10.51 |
| Labor | 622,987 | 541,015 | 704,959 | 1,344,341 | 3,213,302 | 1.85 |
| Consumables | 0 | 0 | 1,633,686 | 680,702 | 2,314,388 | 1.34 |
| Lab/QC/QA | 93,448 | 81,152 | 105,744 | 201,651 | 481,995 | 0.28 |
| Waste Trt/Dsp | 0 | 0 | 0 | 1,709,273 | 1,709,273 | 0.99 |
| Utilities | 974,626 | 525,422 | 1,481,157 | 2,335,921 | 5,317,127 | 3.07 |
| Transportation | 0 | 0 | 0 | 0 | 0 | 0.00 |
| Miscellaneous | 0 | 0 | 0 | 0 | 0 | 0.00 |
| Subtotal | 144,588,533 | 4,263,041 | 9,704,872 | 14,796,097 | 173,352,543 | 100.00 |
| Contribution (%) | 83.41 | 2.46 | 5.60 | 8.54 | 100.00 |  |

The above annual operating cost table was extracted from the Excel version of the Itemized Cost Report (ICR) of this example.

| Raw Material | # Unit Cost | # Annual Amount | Cost |  |
| --- | --- | --- | --- | --- |
| Bulk | ($/kg) | (kg) | ($/yr) | % |
| Starter | 0.82 | 6,599,974 | 5,411,978 | 3.84 |
| Renin | 5.00 | 659,997 | 3,299,987 | 2.34 |
| Salts | 0.05 | 1,412,806 | 70,640 | 0.05 |
| milk | 0.20 | 659,997,360 | 131,999,472 | 93.66 |
| Discrete | ($/Entity) | (# of Entities) | ($/yr) | % |
| Empty Carton | 1.00 | 97,092 | 97,092 | 0.07 |
| Empty Bags | 0.20 | 310,184 | 62,037 | 0.04 |
| TOTAL |  | 140,941,000 | 100 |  |

The above table shows the raw material cost breakdown for this process.  This table was extracted from the Excel version of the Economic Evaluation Report (EER) of this example. Much more detailed breakdowns of various costs (equipment, labor, utilities, waste treatment, etc) are also available in the Economic Impact Report and Itemized Cost Report.  To save space, these additional tables are not reproduced here.

REFERENCES

[1] C.A. Zaror and D.L. Pyle, “Process design: an exercise and simulation examples”, in Chemical Engineering for the Food Industry, Edited by P.J. Fryer, D.L. Pyle and C.D. Rielly, Published in 1997 by Blackie A & P, an imprint of Chapman & Hall, London, ISBN 0 412 49500 7.

[2] Charles E. Morris, “Golden Cheese Company of California”, Chilton’s Food Engineering, March, 1986.